A CMOS Buffer Amplifier with Slew-Rate

Enhancement and Power Saving Techniques

.include D:\ngspice-43_64\Spice64\bin\tsmc_180nm.txt

.option temp=27C

.option noaskquit

* CMOS Amplifier (AC Analysis Focus)

*************** SUPPLY ****************

VDD vdd 0 1.8

*************** INPUT *****************

* AC input for frequency response

Vin in 0 DC 0.9 AC 1

*************** MOS MODELS ************

.model NMOS NMOS (LEVEL=1 VTO=0.7 KP=120u LAMBDA=0.02)

.model PMOS PMOS (LEVEL=1 VTO=-0.7 KP=50u LAMBDA=0.02)

*************** DIFFERENTIAL STAGE ****

M1 n1 in 0 0 NMOS W=20u L=0.18u

M2 n2 ref 0 0 NMOS W=20u L=0.18u

Vref ref 0 0.9

*************** CURRENT MIRROR LOAD ***

M3 n1 n1 vdd vdd PMOS W=40u L=0.18u

M4 n2 n1 vdd vdd PMOS W=40u L=0.18u

*************** GAIN STAGE ************

M5 out n2 0 0 NMOS W=60u L=0.18u

M6 out n2 vdd vdd PMOS W=100u L=0.18u

*************** BIAS CURRENT **********

* FIXED: bias current should not be tied at output node directly

Ibias n2 0 DC 1.3m

*************** LOAD ******************

CL out 0 100p

*************** FEEDBACK **************

Rin in fb 1k

Rf out fb 298k

*************** AC ANALYSIS ***********

.ac dec 100 1 100Meg

.control

run

* Ensure AC plot is selected

setplot ac1

* Gain (Bode magnitude plot)

plot vdb(v(out))

* Optional: linear magnitude

plot mag(v(out))

print i(CL)

plot i(CL)

.endc

.end

.include D:\ngspice-43_64\Spice64\bin\tsmc_180nm.txt

.option temp=27C

* CMOS Buffer Amplifier with Slew Rate Effect

.option noaskquit

* Supply

VDD vdd 0 1.8

* Input (square wave like your graph)

Vin in 0 PULSE(0 1.8 0 1n 1n 2.5u 5u)

* MOS Models

.model NMOS NMOS (LEVEL=1 VTO=0.7 KP=120u LAMBDA=0.02)

.model PMOS PMOS (LEVEL=1 VTO=-0.7 KP=50u LAMBDA=0.02)

* Input stage

M1 n1 in 0 0 NMOS W=20u L=0.18u

M2 n2 ref 0 0 NMOS W=20u L=0.18u

Vref ref 0 0.9

* Current mirror load

M3 n1 n1 vdd vdd PMOS W=40u L=0.18u

M4 n2 n1 vdd vdd PMOS W=40u L=0.18u

* Gain stage

M5 out n2 0 0 NMOS W=60u L=0.18u

M6 out n2 vdd vdd PMOS W=100u L=0.18u

* Slew-rate limiting current source (~1.3 mA)

Ibias out 0 DC 1.3m

* Load capacitor

CL out 0 100p

* Feedback network → gain ≈ 49.5 dB

Rin in fb 1k

Rf out fb 298k

* Analysis

.tran 0.01u 10u

*.ac dec 100 1 100Meg

.control

run

* Transient waveform

plot v(in) v(out)

* Gain plot

plot vdb(v(out)/v(in))

.endc

.end